2. Assembly of vibrating system (excluding vibrating rod)

Lift the mounting base of vibrating head with two hands gently, and make it placed on the vibrating head frame smoothly;

The screw holes on the vibrating head’s bottom panel are aligned with two screw holes of vibrating head’s connecting piece, and the fix screws are screwed separately into the two screw holes;

Then, install the limit wire rope;

Unscrew the screws on the vibrating head;

Unscrew the corresponding screws on the vibrating head frame;

Make the screw on the vibrating head frame pass through one end of the limit wire rope;

Then, tighten the screws on the vibrating head frame;

Now, one end of the limit wire rope has been fixed to the vibration head frame;

Pass the screw, which has just unscrewed from the vibrating head’s bottom panel, through the other end of the limit wire rope;

And screw the screw into the matched screw hole on the vibrating head’s bottom panel;

Tighten the screws by hex wrench;

Unscrew two screws on the dust cover by hex wrench;

Remove the dust cover and set it aside;

Unscrew the locking cap of the vibrating rod;

Lift the vibrating head gently, and screw the vibrating rod’s protective sleeve into the threaded hole

Next Step is to adjust the position of vibrating head frame.

Summary

1. Firstly, connect the vibrating head and its connecting piece.

2. Then, install the limit wire rope.

3. Next, remove the dust cover, and install the vibration rod’s protective sleeve.