4. Monitor Installation

There are two mounting holes of PC monitor on the upper surface of the cabinet. Remove oneof plastic plugs of mounting holes according to monitor installing direction you prefer;

Install the monitor’s mounting panel and screws it inside the cabinet;

Make the monitor stand aligned with the screw protruding from the cabinet, drop the stand, and tighten the clamping screw with hex wrench;

The next step is to mount the monitor to the stand;

Firstly, mount two screws on the upper back of the monitor, screw them in half, and leave the other half space;

Lift the monitor and place the two suspended screws on two half-slot holes on the top of the stand;

Hold the monitor and tighten two screws on the lower back of the monitor;

Then, tighten the four screws separately with hex wrench;

After mounting the monitor stand, adjust the monitor to the normal operating position;

Summary

1. Install the monitor’s mounting panel and screws inside the cabinet;

2. Monitor stand is aligned with the screw protruding from the cabinet, and it is tightened;

3. Two screws on the upper back of the monitor are screwed in half;

4. Place the monitor at the position of the stand’s half-slot holes;

5. Mount all the screws on the back of the monitor.